# Appendix: Generated Rule Tables and Onset Table

Generated by python scripts/export_rule_tables.py from the code; regenerate after changing the configuration.

## Behaviour-to-alarm onset (single behaviour, speed 40 km/h)

| Behaviour (alone) | Severity MODERATE from | LOW tier from | MEDIUM tier from | HIGH tier from |
| --- | --- | --- | --- | --- |
| Eye closure (continuous) | 0.80 s | 0.80 s | 1.50 s | - |
| PERCLOS (60 s window) | 8.0 % | 8.0 % | 14.8 % | - |
| Yawns in 5 min | 1 | 1 | 3 | - |
| Looking away (continuous) | 2.00 s | 2.00 s | 2.00 s | 3.00 s |
| Eyes-off-road share (30 s) | 25.0 % | 24.5 % | 25.0 % | 40.0 % |
| Phone visible (continuous) | 3.00 s | 3.00 s | 3.00 s | 3.00 s |
| Erratic motion (alert driver) | - | 0.44 g | 0.45 g | 0.45 g |

## Fatigue block (27 rules)

| # | eye_closure | yawning | duration | F |
| --- | --- | --- | --- | --- |
| 1 | Low | Low | Low | Low |
| 2 | Low | Low | Moderate | Low |
| 3 | Low | Low | High | Low |
| 4 | Low | Moderate | Low | Moderate |
| 5 | Low | Moderate | Moderate | Moderate |
| 6 | Low | Moderate | High | High |
| 7 | Low | High | Low | High |
| 8 | Low | High | Moderate | High |
| 9 | Low | High | High | High |
| 10 | Moderate | Low | Low | Moderate |
| 11 | Moderate | Low | Moderate | Moderate |
| 12 | Moderate | Low | High | High |
| 13 | Moderate | Moderate | Low | Moderate |
| 14 | Moderate | Moderate | Moderate | Moderate |
| 15 | Moderate | Moderate | High | High |
| 16 | Moderate | High | Low | High |
| 17 | Moderate | High | Moderate | High |
| 18 | Moderate | High | High | High |
| 19 | High | Low | Low | High |
| 20 | High | Low | Moderate | High |
| 21 | High | Low | High | High |
| 22 | High | Moderate | Low | High |
| 23 | High | Moderate | Moderate | High |
| 24 | High | Moderate | High | High |
| 25 | High | High | Low | High |
| 26 | High | High | Moderate | High |
| 27 | High | High | High | High |

## Distraction block (9 rules)

| # | gaze | phone | D |
| --- | --- | --- | --- |
| 1 | Low | Low | Low |
| 2 | Low | Moderate | Moderate |
| 3 | Low | High | High |
| 4 | Moderate | Low | Moderate |
| 5 | Moderate | Moderate | High |
| 6 | Moderate | High | High |
| 7 | High | Low | High |
| 8 | High | Moderate | High |
| 9 | High | High | High |

## Context block (9 rules)

| # | speed | motion | C |
| --- | --- | --- | --- |
| 1 | Low | Low | Low |
| 2 | Low | Moderate | Moderate |
| 3 | Low | High | High |
| 4 | Moderate | Low | Moderate |
| 5 | Moderate | Moderate | Moderate |
| 6 | Moderate | High | High |
| 7 | High | Low | Moderate |
| 8 | High | Moderate | High |
| 9 | High | High | High |

## Overall block (27 rules)

| # | fatigue | distraction | context | R (alarm tier) |
| --- | --- | --- | --- | --- |
| 1 | Low | Low | Low | NONE |
| 2 | Low | Low | Moderate | NONE |
| 3 | Low | Low | High | HIGH |
| 4 | Low | Moderate | Low | LOW |
| 5 | Low | Moderate | Moderate | MEDIUM |
| 6 | Low | Moderate | High | HIGH |
| 7 | Low | High | Low | MEDIUM |
| 8 | Low | High | Moderate | HIGH |
| 9 | Low | High | High | HIGH |
| 10 | Moderate | Low | Low | NONE |
| 11 | Moderate | Low | Moderate | LOW |
| 12 | Moderate | Low | High | HIGH |
| 13 | Moderate | Moderate | Low | LOW |
| 14 | Moderate | Moderate | Moderate | MEDIUM |
| 15 | Moderate | Moderate | High | HIGH |
| 16 | Moderate | High | Low | MEDIUM |
| 17 | Moderate | High | Moderate | HIGH |
| 18 | Moderate | High | High | HIGH |
| 19 | High | Low | Low | LOW |
| 20 | High | Low | Moderate | MEDIUM |
| 21 | High | Low | High | HIGH |
| 22 | High | Moderate | Low | MEDIUM |
| 23 | High | Moderate | Moderate | HIGH |
| 24 | High | Moderate | High | HIGH |
| 25 | High | High | Low | MEDIUM |
| 26 | High | High | Moderate | HIGH |
| 27 | High | High | High | HIGH |